Beckhoff PLC Commissioning document

Christopher Townswick

Helpful info:

AS2021-0d20 is compatible with TE5910 motion designer

TC2_MC2 is currently implemented drive function blocks

TC3_drivemotioncontrol is another option - similar function blocks except:

1.Mc_home has no bcalibrationcam input - must be set at CoE object

0x8n61 "DMC Features Ch. n".

2. Motion fbs need acceleration inputs, not default

(from el7062 doc)

Function does not retain variables

Function block and program both do

Program can only have once instance

Function block must be instantiated and called in program

Axis move command cannot have 0 velocity even if target position is unchanged

Safe z is the lower limit for z-> can only go below while there is no xy motion

Alternative to safe z is z auto with feedback:

Skim water surface -> feedback from massa, sonar for daq

Follow sediment -> feedback from sonar, sonar for daq

250us cycle time:

https://infosys.beckhoff.com/english.php?content=../content/1033/cx7000/10805806219.html

Google search site:infosys.beckhoff.com

DUT data unit type

POU program organization unit

CoE CANOpen over Ethernet

TwinCat 3 version used: 4026.18

Relevant TwinCat packages:

TC1000 adc communication spec

TE5950 drive manager

TF1800 HMI client for integrated VISU/ test controls

TE2000 HMI design interface and client

TF2000 HMI server

Hardware:

CX5340-0195 PLC – Access BIOS hold Del

EL7062 stepper/servo motor terminals

EL3174-0090 -> analog input 16 bit resolution v/I -> minimum cycle time 500us

PLC project properties visualization profile

PDO process data object

Must be in config mode to scan hardware in i/o manager

As2021-0d20 commissioning:

Field Oriented Control (FOC) wake and shake

Log out save all

Wire all drives, in config mode I/O, Devices, right click scan

Encoder high signal any more than 0.5v any less than -0.5

Stepper FOC mode 4096 increments less current legacy mode max current

Encoder cable wiring note:

8 pin m12 cable – pin 1/ red (tensility cable) must be wired to 0v, el7062 input 9

Pin 2/yellow must be wired to 5v, el7062 input 1. This is the opposite of Beckhoff documentation for el7062 terminal

Don’t touch current controller calc by drive manager 2

Hook up to load, feed constant

Cant have 2 of same mc function block (eg mc_moveabsolute) in same program

Software:

Function Block (FB): instantiated in program variables. Can instantiate multiple. Retains data

Program(PRG): Called directly. Can assign task. Only one instance

Function: does not retain data after call

Method: subordinate to a FB or PRG, can access all data

Function Block Diagram (FBD): graphical arrangement of function blocks

Other options – Ladder Diagram, Instruction List

String functions FIND2, LEN2 etc used for strings longer than 255 char

MQTT/Libraries:

MQTT QOS currently set to at most once message send

Mqtt is in‘TC3_IotBase’

LEN2() is in library ‘Tc2_Utilities’

JSON functions in ‘TC3_JsonXML’

FB_S_UPS_BAPI_0 (write persistent data on power loss) in library Tc2_SUPS (download in pkg manager)

Measure Timing Function:

VAR
    MeasureStart    : T_DCTIME64;
    MeasureResult   : T_DCTIME64;
END_VAR

PROGRAM:
MeasureStart:=F_GetActualDcTime64();

(*Insert your program code to measure the processing time*)
MeasureResult:=F_GetActualDcTime64()-MeasureStart;

Drive Manager 2 Settings:

Scaling Nc parameters invert both for coupled axis

Axis Settings:

MOTION -> Axis -> parameter -> other settings -> data persistence ‘true’ ALSO NC-Task SAF -> settings -> load/store

Reverse axis motion direction

MOTION -> Axis ->  Enc -> Encoder Evaluation -> Invert Encoder Counting ‘true’

MOTION -> Axis -> Drive -> Output Settings -> Invert Motor Polarity ‘true’

I/O Settings:

Terminal el7062->coe online tab->8010:39 ‘select info data 1’ data type digital inputs

8110:39 ‘select info data 1’ data type digital inputs

Under i/o manager, el7062 terminal dropdown menu, drv infodata 1 channel 1, info data 1, link to.. -> plc variable in this format 		‘InfoData1Ch1 AT %I*		:UINT;’

Then do the same with drv infodata 1 channel 2

The bits of these uint variables can be read as BOOL e.g. ‘xLimPos := InfoData1Ch1.0;’ (xLimPos is a BOOL type variable)

Terminal el7062->coe online tab->8001:0 ‘FB Touch Probe Settings Ch.1’

->8001.11 ‘Touch Probe 1 Source’ set to ‘Hardware Zero Impulse’

->8101.11 ‘Touch Probe 2 Source’ set to ‘Hardware Zero Impulse’

Terminal el7062->process data tab->sync manager window ‘outputs’

In the PDO assignment window set 0x1607 ‘FB Touch Probe Control Channel 1’

And 0x1687 ‘FB Touch Probe Control Channel 2’

Sync manager window ‘inputs’

In the PDO assignment window set:

0x1A07 ‘FB Touch Probe Status Channel 1’

0x1A08 ‘FB Touch Probe 1 pos position Channel 1’

0x1A87 ‘FB Touch Probe Status Channel 2’

0x1A88 ‘FB Touch Probe 1 pos position Channel 2’

These settings give the touch probe the zero sync pulse from the encoders for both channels. Need to reset linked axes in drive manager 2 as well as plc variable links for

drive info data 1 ch.1 and ch.2

for the relevant axes, solution explorer->motion->axes->’axis’->enc->parameter->homing

‘Invert Direction for Homing Sensor Search’ set to ‘TRUE’

‘Invert Direction for Sync Impulse Search’ set to ‘FALSE’

‘Reference Mode (Sync Condition)’ set to ‘Hardware Sync (feedback reference pulse)’ (‘default’ backs off switch set amount)

Taken from infosys->fieldbus components->ethercat plug-in modules->ej7411

->ej7411 – commissioning->commissioning with the twincat nc->homing

->switching signal from a digital input of the ej7411

Massa Settings:

No echo set to 2mA output -> 0 output from EL terminal

TODO:

Carriage offsets to massa head and keyence head

Plot data

Test keyence and axis daq accuracy

Test mqtt control for daq with both int and bool ctr signals

Header – script name, sensor, sensor offsets, trigger type, continuous mode, number script runs, acquire every x

(temperature?)

(For each sensor – attached to x or xy or xyz or none?)

Data – pass number, pass runtime, timestamp, x,y,z, sensor

Mqtt broadcast topic for all status variables

Documentation on all variables and read/write

Homing routine – 2 switches, drive coupled. Detect which is hit first or if they are both hit.

Then uncouple and drive the side not hit to its switch. Then home routine. This eliminates rack

Mqtt send data at each trigger

Stream data over mqtt with error correction

Choose to display coordinates in terms of sensor offsets

Force no movement for time based daq trigger

Just ping option for each sensor with graphic

Calculate number of data points to be collected by script, display in HMI and disallow overflows

Lock control panel during run

Add checks that motion commands do not violate stop locations

Real time position set point generation for z axis - MC_ExtSetPointGenEnable

VAR_GLOBAL PERSISTENT

nVarPers1 : DINT; (* 1. Persistent variable *)

bVarPers2 : BOOL; (* 2. Persistent variable *)

END_VAR

Write using - FB_WritePersistentData

Cart space – hard stops persistent

Parallel marks on x to assist in squaring manually

Define basin – put in header, clear script? Should entered coordinates be in basin space or normal? Output data file to basin coordinates

Potential Features:

Axis reset -> homing routine

Individual axis reset

Strain gauge on mast tied to estop

DAQ Description:

200,000 point buffer for position, time, and sensor data

Any move instruction becomes daq instruction by selecting a trigger(any axis or time based)

Select any number of sensors to collect for a given daq instruction

Name of script determines folder name (inside C:\data) that will contain saved script and data files

DAQ data is written after the final DAQ instruction in a script

Software Description:

Continuous mode – override append mode – force new file each run – pass index resets for each file

Append mode – pass index keeps incrementing and resets at the end of a finite number of script runs

Basin Must be set – sets boolean true

Boolean must be true to run script

Basin sets folder in /data/ for all scripts and data files

Optional Boolean to display and record coordinates in basin or absolute values

Script is checked for all motion instructions to be inside basin

ScriptRead loads basin that was associated with script originally

Documentation:

Set processor cores

Rescan hardware e bus

Reset drive manager hw and reestablish variable link for latch encoder inputs

HMI/External Variable Description:

| Topic | Variable Name | Type | Read/Write | Description |
| --- | --- | --- | --- | --- |
| MOTION/STATUS | MoveActive | BOOL | Read | Set high when any axis is in motion. Not set by jogging motion |
| MOTION/STATUS | eStop | BOOL | Read | Set high when physical estop button is activated |
| MOTION/STATUS | Script_Open_Err | BOOL | Read | Set high when opening a script file fails. Incorrect path or file does not exist |
| MOTION/STATUS | Script_Read_Err | BOOL | Read | Set high when reading a script file into program fails. Incorrect formatting |
| MOTION/STATUS | Move_Rel_Err | BOOL | Read | Set high when a move relative command would send carriage out of bounds |
| MOTION/STATUS | Move_Abs_Err | BOOL | Read | Set high when a move absolute command would send carriage out of bounds |
| MOTION/STATUS | Basin_Set_Err | BOOL | Read | Set high when attempting to create a basin max lower than the basin min |
| MOTION/STATUS | HomeDone | BOOL | Read | Set high when the homing routine successfully completes. Required for script to run |
| MOTION/STATUS | HardStopLoc | BOOL | Read | Set high when hard stop locate routine successfully completes. Required for script to run |
| MOTION/STATUS | zSafeSet | BOOL | Read | Set high when a safe height for z is set. Script motion won’t allow z to travel below this level. Required for script to run |
| MOTION/STATUS | BasinSet | BOOL | Read | Set high when a basin has been set. Required for script to run and script write |
| MOTION/STATUS | HaltDone | BOOL | Read | Set high when halt command is sent. Script motion and and daq are paused until AXESHALT is set low |
| MOTION/STATUS | PowerError | BOOL | Read | Set high if there is a power error on any motor |
| MOTION/STATUS | CurInstr | INT | Read | Index for script array. Display current script instruction |
| MOTION/STATUS | NextInstr | INT | Read | Used to display next script instruction |
| MOTION/STATUS | RUN_INDEX | INT | Read | Current script run index. Incremented at the end of script run, reset when equal to SCRIPTITERATIONS |
| MOTION/STATUS | xCoupled | BOOL | Read | Set high if 2 x axes are coupled together. Required for script to run |
| MOTION/STATUS | xPosOut | LREAL | Read | x Axis position including offset to carriage from rail OR position within basin |
| MOTION/STATUS | xVelOut | LREAL | Read | x Axis velocity |
| MOTION/STATUS | yPosOut | LREAL | Read | y Axis position including offset to carriage from rail OR position within basin |
| MOTION/STATUS | yVelOut | LREAL | Read | y Axis velocity |
| MOTION/STATUS | zPosOut | LREAL | Read | z Axis position including offset to carriage from rail |
| MOTION/STATUS | zVelOut | LREAL | Read | z Axis velocity |
| MOTION/STATUS | HardStopLocations[] | LREAL | Read | 3 point array of end stops. X,y,z axis |
| MOTION/CTR | CMDADD | BOOL | Write | Add INSTR_BUFFER to script at index CurInstr, CurInstr + 1 |
| MOTION/CTR | CMDCLR | BOOL | Write | Clear script, CurInstr = 0 |
| MOTION/CTR | CMDDEL | BOOL | Write | Delete most recent instruction, CurInstr - 1 |
| MOTION/CTR | CMDRUN | BOOL | Write | Run script. Requires BasinSet, HardStopLoc, HomeDone, zSafeSet, xAxisCoupled all true |
| MOTION/CTR | CMDHOME | BOOL | Write | Run homing routine, set HomeDone true |
| MOTION/CTR | CMDSETZLIM | BOOL | Write | Set safe z lower limit, set zSafeSet true |
| MOTION/CTR | CMDFINDSTOPS | BOOL | Write | Run hard stop locate routine, set HardStopLoc true |
| MOTION/CTR | CMDLOOP | BOOL | Write | If true script will repeat indefinitely, data forced to new file each time, RUN_INDEX will increment indefinitely |
| MOTION/CTR | AXESHALT | BOOL | Write | Pause axis motion. Set low to continue |
| MOTION/CTR | AXESEN | BOOL | Write | Enable to power all axes |
| MOTION/CTR | AXESRESET | BOOL | Write | Set true to reset all axes in case of error or crash |
| MOTION/CTR | CMDSCRIPTWRITE | BOOL | Write | Write script to file in c/data/basin/ |
| MOTION/CTR | CMDSCRIPTREAD | BOOL | Write | Read script based on basin name and script name. BasinSet must be true |
| MOTION/CTR | SCRIPTITERATIONS | INT | Write | Number of times script will repeat when CMDRUN set true |
| MOTION/CTR | SCRIPTNAME | STRING | Write | Used by script read and write |
| MOTION/CTR | BASINNAME | STRING | Write | Enter basin name. basin name not set until BasinSet is true |
| MOTION/CTR | USEBASINCOORDINATES | BOOL | Write | Display coordinate outputs in terms of set basin |
| MOTION/CTR | BASINCTR | BOOL | Write | Control to set basin limits. Select |
| MOTION/CTR | DEFAULTBASIN | BOOL | Write | Control to set basin to maximum limits based on hard stops |
| MOTION/CTR | XJOGFW | BOOL | Write | Directly control manual motion |
| MOTION/CTR | XJOGBW | BOOL | Write | Directly control manual motion |
| MOTION/CTR | YJOGFW | BOOL | Write | Directly control manual motion |
| MOTION/CTR | YJOGBW | BOOL | Write | Directly control manual motion |
| MOTION/CTR | ZJOGFW | BOOL | Write | Directly control manual motion |
| MOTION/CTR | ZJOGBW | BOOL | Write | Directly control manual motion |
| MOTION/INSTR_BUFFER | MotionCommand | INT/ENUM | Write | Type of motion instruction 0: null instruction 1: waitok – activates AXESHALT, waits for button click before continuing script 2: absolute – motion based on absolute coordinates 3: relative – motion relative to current position |
| MOTION/INSTR_BUFFER | zType | INT/ENUM | Write | Z axis behavior 0: manual – z motion controlled manually 1: watersurface – z automatically tracks water surface. Not implemented 2: sedsurface – z automatically tracks sediment surface. Not implemented |
| MOTION/INSTR_BUFFER | DataTrigger | INT/ENUM | Write | Determines what activates data recording. Resolution is determined by DAQRES 0: none – no daq 1: x – daq triggers on x axis 2: y – daq triggers on y axis 3: z – daq triggers on z axis 4: timed – daq triggers on time |
| MOTION/INSTR_BUFFER | Acq_Keyence | BOOL | Write | Keyence data will be recorded |
| MOTION/INSTR_BUFFER | Acq_Massa | BOOL | Write | Massa data will be recorded |
| MOTION/INSTR_BUFFER | Acq_Sonar | BOOL | Write | Sonar data will be recorded. Not implemented |
| MOTION/INSTR_BUFFER | SetXPosition | REAL | Write | Target x position |
| MOTION/INSTR_BUFFER | SetXVelocity | INT | Write | Target x velocity |
| MOTION/INSTR_BUFFER | SetXAccel | INT | Write | Target x acceleration. Not implemented |
| MOTION/INSTR_BUFFER | SetYPosition | REAL | Write | Target y position |
| MOTION/INSTR_BUFFER | SetYVelocity | INT | Write | Target y velocity |
| MOTION/INSTR_BUFFER | SetYAccel | INT | Write | Target y acceleration. Not implemented |
| MOTION/INSTR_BUFFER | SetZPosition | REAL | Write | Target z position. Setting lower than safe z will cause z to move to safe z |
| MOTION/INSTR_BUFFER | SetZVelocity | INT | Write | Target z velocity |
| MOTION/INSTR_BUFFER | PauseTime | INT | Write | Time to pause at the end of motion in milliseconds |
| MOTION/INSTR_BUFFER | Comment | STRING | Write | comment |
| DAQ/DATA | MASSA_TEMP | INT | Read | Real time massa measurement |
| DAQ/DATA | KEYENCE_TEMP | INT | Read | Real time keyence measurement |
| DAQ/CTR | CMDDATACLR | BOOL | Write | Clear all data arrays and reset index |
| DAQ/CTR | CMDDATAAPPEND | BOOL | Write | Set high to save all sensor data to one file for a finite number of script runs |
| DAQ/CTR | CMDDATAWRITE | BOOL | Write | Test function to dump data to file. Script automates this process |
| DAQ/CTR | DAQRES | INT | Write | Resolution of data acquisition. Acquire every x mm/seconds. Default 1 is the highest resolution |