SICK camera control application information

win10 setup - iCon api 5.1

camera SW 5.1.0.14

fpga 1.4.13.0

DISCONNECT FROM RANGER STUDIO OR C++ WILL NOT WORK

Find center of sensor FOV: right now ~col773

Subtract x value of col773 from all x data and multiply by -1 to shift x zero to center of fov

Apply rail offsets and sick offsets to put data in cart space

SICK_Control:

Section_Overlap:

Y

bosch + spacer(6mm):

measured max 945.27mm - min 1.61mm = 943.66mm/2 = 471.83 col773

plumb bob off of center of laser agrees with col773 as center

col 767 Y=468.26mm +

col 768 Y=469.38mm /2= 468.82mm*2=937.64mm

step ladder:

col773 Y=472.2mm

step ladder + short cinderblock:

col773 Y=472.45mm

average midpoint: 472.16mm

Z (fixed on test bed)

step ladder + short cinderblock:

col773 Z=715.29mm

measure from laser Z=-359mm

gap Z=1074.29mm

step ladder:

col773 Z=572.35mm

measure from laser Z=-501mm

gap Z=1073.35

cinderblock:

col773 Z=151.24mm

measure from laser Z=-925mm

gap Z=1076.24mm

average gap: 1074.62mm

cart offsets from rails

x:84.25 y:62.5 z:40

SICK offsets from cart

x:-1.83 y:-23.12 z:-349

characterize warping at edges of field of view to recommend swath overlap for scans